. RETAINING LAYER

Supply and installation of:

DAKU GEO 100, black anti-erosion alveolar structures, made up of three-dimensional ovoid cells, made of polyethylene, with a perforated surface (10 mm holes) to allow drainage, 100 mm high, is necessary in the DAKU stratigraphy for green roofs with inclinations ranging from 15 ° to 25° (+/-15%) to allow the retention of the DAKU ROOF SOIL substrates to an extent of 100 mm "settled", avoiding slipping, counteracting the phenomena of slipping and surface runoff in the absence of widespread and rooted vegetation.

Technical data:

| - Cells height | 100 mm | (typical values) |  |
| --- | --- | --- | --- |
| - Cell length | 287 mm | (+/- 10%) |  |
| - Cell width | 320 mm | (+/- 10%) |  |
| - Tensile Strength | 7,5 kN/m | (EN ISO 9864) |  |
| - Unit size | 2,6 x 9,70 m |  |  |
| - Unit area | 25 sqm |  |  |
| - number of cells/sqm | 21,7 |  |  |

DAKU GEO 100 cells are installed on the DAKU GRID 4 geogrid, to which they are tied with nylon stripes, or directly on the DAKU STABILFILTER substrates, when the shape and slope of the roofs allows it. The cells are then filled with the DAKU ROOF SOIL substrate, gradually starting from the bottom to ensure a proper filling. In order to speed the settlement of the substrate, it is better to wet it.

sqm 0,00