. CONTAINMENT/SEPARATION COMPONENT

Supply and installation of:

DAKU PRO, which is a containment component 8 cm high. It is used for the containment of the DAKU ROOF SOIL substrate or of other components (fillers, blocks, paving) in DAKU’s systems. It is built in an aluminum-magnesium alloy (pressed and folded) with good resistance to corrosion and traction. It has holes for drainage and ventilation (4 mm wide) in compliance with the UNI 11235/2015 standards.

Technical data:

…ml of DAKU PRO 80 - AL

| - Thickness |  | 2,0 mm |
| --- | --- | --- |
| - Wideness |  | 144 mm |
| - Length |  | 2,00 ml |
| - Drainage and ventilation surface | 144 cmq/ml |  |

…ml of DAKU PRO FLEX 80 - AL

| - Thickness | 2,0 mm |
| --- | --- |
| - Wideness | 144 mm |
| - Length | 2,00 ml |
| - Drainage and ventilation surface | 144 cmq/ml |

n° …. pcs of DAKU GL80 – AL linear junction

n° …. pcs of DAKU GA80 – AL corner junction

The components are installed directly on the DAKU STABILFILTER substrates without using nails or bolts. The connections between the components will be locked with the DAKU GL80 linear junction and with the DAKU GA80 special corner junction.

ml 0,00