. INSPECTION ELEMENT

Supply and installation of:

DAKU CONTROLLER, which is an inspection component for ventilation and drainage pipes. The component is high enough to surmount the thickness of the DAKU ROOF SOIL substrate or of the other components of DAKU’s systems. It is made of an aluminum-magnesium alloy, which is pressed, folded and riveted. It has high resistance to corrosion and traction. In compliance with UNI ISO 11235:2015 standards, it has 4 mm holes for drainage and ventilation; it also includes a closable cover with holes.

Technical data:

… pcs of the DAKU CONTROLLER 4F25 – AL

| - Thickness | 2,0 mm |
| --- | --- |
| - Basic body dimensions | 250 x 250 x 100 mm |
| - Cover dimensions | 250 x 250 x 20 mm |
| - Maximum extension | 400 x 400 mm ca (4 supporting flanges) |
| - Drainage and ventilation holes (dim.25x4 mm) | n. 112 (basic body) + n. 58 (cover) |
| - Drainage and ventilation surface | 170 cmq (cover included) |

… pcs of the DAKU CONTROLLER 3F25 – AL

| - Thickness | 2,0 mm |
| --- | --- |
| - Basic body dimensions | 250 x 250 x 100 mm |
| - Cover dimensions | 250 x 250 x 20 mm |
| - Maximum extension | 330 x 400 mm ca (3 supporting flanges) |
| - Drainage and ventilation holes (dim.25x4 mm) | n. 112 (basic body) + n. 58 (cover) |
| - Drainage and ventilation surface | 170 cmq (cover included) |

… pcs of the DAKU CONTROLLER 2F25 – AL

| - Thickness | 2,0 mm |
| --- | --- |
| - Basic body dimensions | 250 x 250 x 100 mm |
| - Cover dimensions | 250 x 250 x 20 mm |
| - Maximum extension | 330 x 330 mm ca (2 supporting flanges) |
| - Drainage and ventilation holes (dim.25x4 mm) | n. 112 (basic body) + n. 58 (cover) |
| - Drainage and ventilation surface | 170 cmq (cover included) |

… pcs of the DAKU CONTROLLER 3F25 SLIM – AL

| - Thickness | 2,0 mm |
| --- | --- |
| - Basic body dimensions | 250 x 250 x 50 mm |
| - Cover dimensions | 250 x 250 x 20 mm |
| - Maximum extension | 330 x 400 mm ca (3 supporting flanges) |
| - Drainage and ventilation holes (dim.25x4 mm) | n. 96 (basic body) + n. 58 (cover) |
| - Drainage and ventilation surface | 154 cmq (cover included) |

… pcs of the DAKU CONTROLLER 2F25 SLIM – AL

| - Thickness | 2,0 mm |
| --- | --- |
| - Basic body dimensions | 250 x 250 x 50 mm |
| - Cover dimensions | 250 x 250 x 20 mm |
| - Maximum extension | 330 x 330 mm ca (2 supporting flanges) |
| - Drainage and ventilation holes (dim.25x4 mm) | n. 96 (basic body) + n. 58 (cover) |
| - Drainage and ventilation surface | 154 cmq (cover included) |

… pcs of DAKU RING 50 P25 – AL extension

| - Thickness | 2,0 mm |
| --- | --- |
| - Height | 50 mm |
| - Dimensions | 250 x 250 mm ca |

… pcs of DAKU RING 75 P25 – AL extension

| - Thickness | 2,0 mm |
| --- | --- |
| - Height | 75 mm |
| - Dimensions | 250 x 250 mm ca |

… pcs of DAKU RING 100 P25 – AL extension

| - Thickness | 2,0 mm |
| --- | --- |
| - Height | 150 mm |
| - Dimensions | 250 x 250 mm ca |

… pcs of DAKU RING 200 P25 – AL extension

| - Thickness | 2,0 mm |  |
| --- | --- | --- |
| - Height | 200 mm |  |
| - Dimensions | 250 x 250 mm ca |  |

Installation without fixing to the structure, by positioning on the DAKU DRAIN, FSD or HYPERDRAIN drainage and water storage elements on which an opening is made sufficient to ensure that the drain outlet is reached; subsequently the stabilization and DAKU filter layers will be turned up on the side walls up to the filling level of the DAKU ROOF SOIL substrate or gravel.

pcs 0,00